Write a program to perform 8 * 8 multiplication by successive addition.

| Address | Assembly Code | Hex Code | Comments |
| --- | --- | --- | --- |
| 0000 | LXI H, 2050H | 21 50 20 | Point to get count of numbers |
| 0003 | MOV C, M | 4E | Move Multiplicand to C |
| 0004 | INX H | 23 | Increase H to point to Multiplier |
| 0005 | MOV B, M | 46 | Move Multiplier to B |
| 0006 | XRA A | AF | A <- A XOR A to clear A |
| 0007 | MVI D, 00H | 16 00 | Move Immediate 00 to D |
| 0009 | LOOP: ADD B | 80 | A <- A+B |
| 000A | JNC NEXT | D2 0E 00 | Jump to Label Next if no carry |
| 000D | INR D | 14 | Increase D |
| 000E | NEXT: DCR C | 0D | Decrease C by 1 |
| 000F | JNZ LOOP | C2 09 00 | Jump to Label loop if C not Zero |
| 0012 | INX H | 23 | Increase H by 1 |
| 0013 | MOV M, A | 77 | Move data in Accumulator to Memory |
| 0014 | INX H | 23 | Increase H by 1 |
| 0015 | MOV M, D | 72 | Move data in reg D to Memory |
| 0016 | HLT | 76 | Halt |

Procedure:

Step – 1: Writing program in memory.

- Press Reset

- Press SET/MEM

- Type in Address 0000

- Press Enter

- Type 1st Hex Code (Here 21)

- Press Enter

- Follow step 5 and 6 to type in all Hex Code

Step – 2: Assigning Value to the Address Location

- Press Reset

- Press SET/MEM

- Type in Address of 1st Location (Here 2050)

- Press Enter

- Enter value of Multiplicand

- Press Enter

- Enter value of Multiplier

- Press Enter

Step – 3: Executing the program

- Press Reset to clear Buffer

- Press Go

- Enter Starting Address of Program (Here 0000)

- Press Execute

Step – 4: Checking output

- Press reset and Clear the Buffer

- Press Go

- Enter Result Location (Here 2052 for LB and 2053 for UB)

- You will find the product of the numbers

Output: