# Write a program in 8085 to multiply two 8-bit numbers using shifting multiplicand.

| Address | Assembly Code | Hex code | Comments |
| --- | --- | --- | --- |
| 0000 | LHLD 2050H | 2A 50 20 | Loading data to HL reg pair |
| 0003 | MOV E,L | 5D | Move data in L to E |
| 0004 | MOV A, H | 7C | Move data in H to Acc |
| 0005 | MVI D, 00H | 16 00 | Moving 00 to D |
| 0007 | MVI C, 08H | 0E 08 | Move 8 to C |
| 0009 | LXI H, 0000H | 21 00 00 | Loading 0000 to HL reg pair |
| 000C | BACK: RAR | 1F | Rotate accumulator data to right by 1 bit. |
| 000D | JNC NEXT | D2 11 00 | Jump to next if no carry |
| 0010 | DAD D | 19 | HL <- HL + DE |
| 0011 | NEXT: XCHG | EB | HL <-> DE |
| 0012 | DAD H | 29 | HL <- HL+HL (left shift) |
| 0013 | XCHG | EB | HL <-> DE |
| 0014 | DCR C | 0D | Decrease C by 1 |
| 0015 | JNZ BACK | C2 0C 00 | Jump to BACK if C not zero |
| 0018 | SHLD 2052 | 22 52 20 | Store HL reg pair to 2052 and 2053 |
| 001B | HLT | 76 | Halt |

Procedure:

Step – 1: Writing program in memory.

- Press Reset

- Press SET/MEM

- Type in Address 0000

- Press Enter

- Type 1st Hex Code (Here 21)

- Press Enter

- Follow step 5 and 6 to type in all Hex Code

Step – 2: Assigning Value to the Address Location

- Press Reset

- Press SET/MEM

- Type in Address of 1st Location (Here 2050)

- Press Enter

- Enter value of Multiplicand

- Press Enter

- Enter value of Multiplier

- Press Enter

Step – 3: Executing the program

- Press Reset to clear Buffer

- Press Go

- Enter Starting Address of Program (Here 0000)

- Press Execute

Step – 4: Checking output

- Press reset and Clear the Buffer

- Press Go

- Enter Result Location (Here 2052 for LB and 2053 for UB)

- You will find the product of the numbers

Output: