# Write a program in 8085 Assembly language to divide a 16-bit number by an 8-bit number.

| Address | Assembly Code | Hex Code | Comments |
| --- | --- | --- | --- |
| 0000 | LHLD 2050 | 2A 50 20 | Load hl pair register from memory location 2050h |
| 0003 | LDA 2052H | 3A 52 20 | Load into Accumulator from memory location 2052h |
| 0006 | MOV B, A | 47 | Move data (divisor) from accumulator to B register |
| 0007 | MVI C, 08H | 0E 08 | Move 8 to C |
| 0009 | BACK: DAD H | 29 | HL <- HL + HL |
| 000A | MOV A, H | 7C | Move data (LB of dividend) in H register to accumulator |
| 000B | SUB B | 90 | Subtract B register value from accumulator |
| 000C | JC NEXT | DA 11 08 | Jump if carry to Label NEXT |
| 000F | MOV H, A | 67 | Move data in accumulator to H register |
| 0010 | INR L | 2C | Increment L register by 1 |
| 0011 | NEXT: DCR C | 0D | Decrease register C by 1 |
| 0012 | JNZ BACK | C2 09 08 | Jump to Label BACK if C not zero |
| 0015 | SHLD 2053H | 22 53 20 | Store HL register pair in memory location 2053h |
| 0018 | HLT | 76 | Halt |

Procedure:

Step – 1: Writing program in memory.

- Press Reset

- Press SET/MEM

- Type in Address 0000

- Press Enter

- Type 1st Hex Code (Here 21)

- Press Enter

- Follow step 5 and 6 to type in all Hex Code

Step – 2: Assigning Value to the Address Location

- Press Reset

- Press SET/MEM

- Type in Address of 1st Location (Here 2050)

- Press Enter

- Enter value of Dividend (LB)

- Press Enter

- Enter value of Dividend (UB)

- Press Enter

- Enter value of Divisor

Step – 3: Executing the program

- Press Reset to clear Buffer

- Press Go

- Enter Starting Address of Program (Here 0000)

- Press Execute

Step – 4: Checking output

- Press reset and Clear the Buffer

- Press Go

- Enter Result Location (Here 2053 for quotient and 2054 for Remainder)

- You will find the product of the numbers

Output: