Name: Tanmay Vig

Roll No.: 19BCS061

Class: B. Tech 3rd year

Q13: Write and execute a program in the assembly language of 8085 to find HCF of given two integers stored at a given location.

Tool used: 8085 simulator

Program:

| Address | Assembly Code | Hex Code | Comments |
| --- | --- | --- | --- |
| 0000 | LXI H, 2050H | 21 50 20 | Point reg pair HL to memory location 2050H (first Num) |
| 0003 | MOV A,M | 7E | Move memory to Accumulator. |
| 0004 | INX H | 23 | Increase the pointed location by 1 |
| 0005 | MOV B,M | 46 | Move memory to reg B |
| 0006 | LOOP: CMP B | B8 | Compare B with A |
| 0007 | JZ STORE | CA 17 00 | If both are equal then store the result |
| 000A | JC EXG | DA 00 11 | If B>A, then swap values |
| 000D | SUB B | 90 | If B<A, subtract B from A |
| 000E | JMP LOOP | C3 06 00 | Jump to Label LOOP |
| 0011 | EXG: MOV C,B | 48 | Move value in reg B to reg C |
| 0012 | MOV B,A | 47 | Move value in Accumulator to reg B |
| 0013 | MOV A,C | 79 | Move value in reg C to Accumulator |
| 0014 | JMP LOOP | C3 06 00 | Jump to Label LOOP |
| 0017 | STORE: STA 2052H | 32 52 20 | Store value of Accumulator in memory location 2052H |
| 001A | HLT | 76 | Halt |

| Address | Data |
| --- | --- |
| 2050H | 2D |
| 2051H | 69 |
| 2052H | 0F |

Input:

Output: